How to charge VistaZ battery?

v1.0

Contents

1.	Pre-Requisite	3

2.	Battery charging limit for Air travel	4

3.	How to charge the 18650 battery without defined charging current	4

4.	Li-ion Battery Mode Selection	5

5.	Setting the charging current to desired level	6

6.	Battery Charging Status	7

### Pre-Requisite

  - Nitecore UM4 battery charger – 1 qty

  - A Male to Micro B cable – 1 qty

  - Howell 2500mAH/3000mAh 18650 batteries – 1 qty

  - USB wall adapter – 1 qty

### Battery charging limit for Air travel

18650 batteries should not be fully charged but can be 70% only during the Air travel as per the Air shipment guidelines. So when batteries needs to ship through Air to another location, can be charged till 3.85v/4.0v(3.85v for 2500mAh batteries and 4.0 for 3000mAh batteries. Else batteries can be fully(4.2v) charged for local uses/deployment.

### How to charge the 18650 battery without defined charging current

  - Plug the Micro USB cable into Nitecore UM4 charger, without battery it will look like as below

  - If you want to skip the charging current value settings, by inserting the 18650 battery into slot, it will be charge by default charging settings.

  - Below image showing charging time in right and charging current in mA in middle

  - Make sure to turn off charger when battery shows 3.85v/4.0v(Based on #2 topic)

### Li-ion Battery Mode Selection

  - On Nitecore UM4 battery charger, you have two tactile button next to written “UM4”

  - On the Left with engraved letter “C” and on the right with engraved letter “V”

  - Press “C” and hold for 2 sec to enter the charging setting mode

  - Press “C” again to enter battery type selection

  - Current battery type will start flashing

  - By pressing “V”, change to Li-ion, 4.2v

  - Once the values are finalized, long press “C” button to save the value and exit from battery type selection mode

### Setting the charging current to desired level

  - Press “C” button and hold for 3 sec to enter the charging setting mode

  - Charging current value will start flashing

  - By pressing “V”, numeric value can be increased/decreased

  - Once the values are finalized, long press “C” button to save the value and exit from charging setting mode

### Battery Charging Status

  - When battery is charging, screen information will be updating based on current status

    - Charging time(HH:MM)

    - Battery present voltage

    - mAh gained/charged

    - Charging current

    - Battery health

    - Battery internal resistance

    - Battery mounted in slot number

    - Charging mode

  - By pressing “V” charging status display can be checked for specific screen

  - Backlight of the screen will be dimmed after certain period of time